| Use | Product Name | Type | Contact Time | Use Rate [%] |  | Cleaning Tools Required |
| --- | --- | --- | --- | --- | --- | --- |
| Clean | Mains Water | NA | 10 mins | Na |  | Jet Stream ¦ Pad + Bucket ¦ Rinse Gun |

| Recommended Personal Protective Equipment |  | Other Controls |
| --- | --- | --- |
|  |  | ISOLATE & SEGREGATE |

Special Precautions

- NA

| Daily Cleaning Method | Weekly Cleaning Method |
| --- | --- |
| - Prior to commencing any action, make sure you have read the HOPPER CLEAN and WATER HOSE Risk Assessments and make sure you understand the equipment hazards. - Remove the bolts to allow access to the outlet flap panel. - Place the bolts in a secure location. - Remove the outlet flap panel exposing the outlet grid. - Roll the vessel to the cleaning area - Place a bucket under the outlet opening - Using a dustpan brush, brush all of the loose material debris from the internal walls, into the waste bucket - Using the mains hose, wash down the vessel with mains water, paying particular attention to the key inspection points. - Using the mains hose, wash down the outlet flap panel - Hand clean all accessible surfaces of the vessel, including exterior of the vessel, using a damp cloth. - Visually inspect to ensure all surfaces are clean and free from debris. Reclean is necessary. - Leave vessel to dry naturally. - Place vessel back in its designated storage location and put outlet flap panel back in and bolt shut - Label the waste and store it in the waste storage unit | Not required. |
| Key Inspection Points |  |
| - Exterior of vessel - Interior and Outlet Grid | - Outlet flap slot - Outlet flap panel |

| Key Inspection Point 1 | Key Inspection Point 2 |
| --- | --- |
| Key Inspection Point 3 | Key Inspection Point 4 |